Object Type:	freq_monitor

Description:	Calculates running event (spike) frequency over a given time

window using exponential weighting.

Author:		M. Wilson, Caltech (2/89), modified by D. Jaeger

------------------------------------------------------------------------------

ELEMENT PARAMETERS

DataStructure:	freq_mon_type  [in src/device/dev_struct.h]

Size:		84 bytes

Fields:		tmin		minimum time interval which must be held by

the buffer

size            buffer size, in number of events

current  	current buffer location

start           starting buffer location - 1

end  		last occupied buffer location

event_size  	size of each event data structure

event           circular event buffer

exponent	power by which to weight

frequency       curent calculated value of the frequency

(set to zero on reset)

------------------------------------------------------------------------------

SIMULATION PARAMETERS

Function:	FrequencyMonitor  [in src/device/new_frequency.c]

Classes:	device

Actions:	PROCESS  RESET  CREATE

Messages:	NONZERO_TRIG	input 	trigger on nonzero input

ZERO_TRIG	input	trigger only when input is zero

POSITIVE_TRIG	input   trigger on positive input

NEGATIVE_TRIG	input	trigger on negtive input

WINDOW		tmin	sets the window size tmin

CONTINUOUS	input	triggers every time

------------------------------------------------------------------------------

Notes:		This is a frequency measuring device which computes the

running event frequency over a time interval using exponential

weighting.  The field "tmin" should be set in order to specify

the interval or "window" for averaging.  Larger values give a

smoother result.  If "exponent" is unequal to the default

value of zero, inputs are weighted by exp(exponent*time).  The

frequency field may be examined or sent to a graph.

The first four messages are used to give it an input, and the

counting of input events is triggered by one of the four

conditions.  The most common way to use the frequency monitor

is to set the "tmin" field to an appropriate timing window,

send it a POSITIVE_TRIG message with the state of a spiking

element, and send a "PLOT frequency" message from the monitor

to a graph.

Example:	See the demonstration script Scripts/examples/spike/freqmon.g.

See also: